# Task Title: Automotive Codes for Troubleshooting

# OALCF Cover Sheet – Practitioner Copy

Learner Name: __________________________________________

Date Started: ___________________________________________

Date Completed: _________________________________________

| Goal Path: | Employment | Apprenticeship |
| --- | --- | --- |
| Secondary School | Post Secondary | Independence |

Successful Completion:  Yes  	     No

Task Description: Understand diagnostic codes used in automobiles.

Main Competency/Task Group/Level Indicator:

- Find and Use Information/Read continuous text/A1.1

- Find and Use Information/Interpret documents/A2.2

- Use Digital Technology/D.2

Materials Required:

- Pen/pencil and paper

- Computer or digital device

# Learner Information

Most cars today have built-in on-board diagnostics tools (OBD) that can recognize problems and report them using a series of error codes. Automotive Service Technicians must know how to look up error codes to find out what they mean. There are error code databases and charts that provide descriptions of each error code, and most can be found on the internet.

Scan the “Trouble Codes Chart”.

Trouble Codes Chart

| First Character - System | Second Digit - Code Type | Third Digit - Sub-System |
| --- | --- | --- |
| P = Powertrain | 0 = Generic (used by many manufacturers) | 0, 1 and 2 = air/fuel mixture |
| B = Body | 1 = Enhanced (used only by a few Manufacturers) | 3 = ignition system |
| C = Chassis |  | 4 = checking auxiliary emissions |
| U = Network & Vehicle Integration |  | 5 = engine idling |
|  |  | 6 = onboard computer and ancillary outputs |
|  |  | 7, 8 and 9 = transmission (gearbox) |
|  |  | A, B and C = hybrid propulsion |

# Work Sheet

Task 1: An Infiniti has the trouble code B1049.  What do the first three characters and digits in the code represent?

Answer:

______________________________________________________

Task 2: Open a new page of the web browser of the computer.  Search for “Infiniti B1049”.  What does this code indicate as the possible problem?

Answer:

______________________________________________________

Task 3: Conduct an internet search for the following error codes.  Describe the problem each code is reporting.

P0460:

P0169:

P0636:

P0811:

Answer:

______________________________________________________

Task 4: Are the codes in Task 3 “Generic” (used by many manufacturers) or “Enhanced” (used only by a few manufacturers)? How do you know?

Answer:

______________________________________________________

# Answers

Task 1: An Infiniti has the trouble code B1049.  What do the first three characters and digits in the code represent?

Answer:

B = Body

1 = Enhanced (only used by a few Manufactures)

0 = air/fuel mixture

Task 2: Open a new page of the web browser of the computer.  Search for “Infiniti B1049”.  What does this code indicate as the possible problem?

Answer: Driver Air Bag Module Open

Task 3: Conduct an internet search for the following error codes.  Describe the problem each code is reporting.

P0460:

P0169:

P0636:

P0811:

Answer:

P0460: Fuel Level Sensor Circuit Malfunction

P0169: Incorrect Fuel Composition

P0636: Power Steering Control Circuit Low

P0811: Excessive Clutch Slippage

Task 4: Are the codes in Task 3 “Generic” (used by many manufacturers) or “Enhanced” (used only by a few manufacturers)? How do you know?

Answer:  Generic. The second character/digit in the code is a “0”. According to the Trouble Codes Chart, a 0 means generic.

# Performance Descriptors

| Levels | Performance Descriptors | Needs Work | Completes task with support from practitioner | Completes task independently |
| --- | --- | --- | --- | --- |
| A1.1 | reads short texts to locate a single piece of information |  |  |  |
|  | follows simple, straightforward instructional texts |  |  |  |
| A2.2 | performs limited searches using one or two search criteria |  |  |  |
|  | extracts information from tables and forms |  |  |  |
|  | uses layout to locate information |  |  |  |
|  | makes low level inferences |  |  |  |
| D.2 | selects and follows appropriate steps to complete tasks |  |  |  |
|  | locates and recognizes functions and commands |  |  |  |
|  | makes low-level inferences to interpret icons and text |  |  |  |
|  | begins to identify sources and evaluate information |  |  |  |
|  | performs simple searches using keywords (e.g. internet, software help menu) |  |  |  |

This task: Was successfully completed        Needs to be tried again

Learner Comments:

Instructor (print):						Learner (print):

__________________                                         _________________